Exercise 1: flip flops inference

- Drawings

- Arch 1

- Arch 2

- Arch 3

- RTL VIEW

Exercise 2: guess the circuit

  - Line 14, as it declares a variable which needs a register/flip flop to store its value.

  - Line 12, as the signal will need a register/flip flop to store its value

- The hardware will not change, it will need the same number of registers.

- In lieu of reproducing the sketch, I know the y will be offset by one clock in code2 because signal doesn’t update till the next clock cycle unlike the variable y in code1.

- Compile codes and compare

- Simulation comparison

- 4 registers

Exercise 3: leading-ones counter

- VHDL code

  - LIBRARY ieee;

  - USE ieee.std_logic_1164.all;

  - use ieee.numeric_std.all;

  - USE ieee.math_real.all;

  - ----------------------------

  - ENTITY counter_leading_ones IS

  - GENERIC (

  - -- number of input bits

  - BITS: INTEGER := 8

  - );

  - PORT (

  - -- input in stl format

  - x: IN STD_LOGIC_VECTOR(BITS-1 DOWNTO 0);

  - --num of bits, take the log base 2, then convert to an int.

  - y: OUT STD_LOGIC_VECTOR((integer(log2(real(BITS)))) DOWNTO 0)

  - );

  - END ENTITY;

  - ARCHITECTURE sequential OF counter_leading_ones IS

  - BEGIN

  - counter: PROCESS(x)

  - variable count: UNSIGNED(y'RANGE);

  - BEGIN

  - count := to_unsigned(0, y'LENGTH);

  - FOR i IN x'RANGE LOOP

  - CASE x(i) IS

  - WHEN '1' => count := count + 1;

  - WHEN OTHERS => EXIT;

  - END CASE;

  - END LOOP;

  - y <= std_logic_vector(count);

  - END PROCESS counter;

  - END ARCHITECTURE;

- Sequential was simpler by far

- Simulation results

Exercise 4: Switch debouncer

- Use a DFF to check that the signal isn’t changing for T_deb time.

- You can precompute the counter size (20 bits) to not need to store t_deb in a register somewhere. Trade off isn’t that good though.

- ~8

- VHDL code

  - LIBRARY ieee;

  - USE ieee.std_logic_1164.ALL;

  - USE ieee.numeric_std.ALL;

  - USE ieee.math_real.ALL;

  - ENTITY switch_debouncer IS

  - GENERIC (

  - --50MHz

  - fclk  : INTEGER := 50_000_000;

  - -- counter size (20 bits gives 20.9ms with 50MHz clock)

  - -- 2^20 * 1/(50MHz) ~= 20ms

  - -- time to debounce in ms

  - t_deb : INTEGER := 20

  - );

  - PORT (

  - clk, pb : IN STD_LOGIC;

  - pb_deb  : OUT STD_LOGIC

  - );

  - END ENTITY switch_debouncer;

  - ARCHITECTURE arch OF switch_debouncer IS

  - --input flip flops

  - SIGNAL flipflops   : STD_LOGIC_VECTOR(1 DOWNTO 0)                                          := (OTHERS => '0');

  - --sync reset to zero

  - SIGNAL counter_set : STD_LOGIC                                                             := '0';

  - -- we need to divide by 1000 after multiplying t_deb by fclk to adjust for t_deb being in ms

  - SIGNAL counter_out : UNSIGNED(INTEGER(CEIL(LOG2(REAL(t_deb * fclk / 1000)))) - 1 DOWNTO 0) := to_unsigned(0, INTEGER(CEIL(LOG2(REAL(t_deb * fclk / 1000)))));

  - BEGIN

  - --determine when to start/reset counter

  - counter_set <= flipflops(0) XOR flipflops(1);

  - PROCESS (clk)

  - BEGIN

  - IF rising_edge(clk) THEN

  - flipflops(0) <= pb;

  - flipflops(1) <= flipflops(0);

  - --reset counter because input is changing

  - IF (counter_set = '1') THEN

  - counter_out <= to_unsigned(0, counter_out'LENGTH);

  - --stable input time is not yet met

  - ELSIF (counter_out(counter_out'LEFT) = '0') THEN

  - counter_out <= counter_out + 1;

  - --stable input time is met

  - ELSE

  - pb_deb <= flipflops(1);

  - END IF;

  - END IF;

  - END PROCESS;

  - END ARCHITECTURE;

- Demo was shown to you in class. Below is the top level code for said demo.

  - LIBRARY ieee;

  - USE ieee.std_logic_1164.ALL;

  - USE ieee.numeric_std.ALL;

  - USE ieee.math_real.ALL;

  - USE work.ALL;

  - ENTITY toplevel IS

  - GENERIC (

  - -- our clock frequency

  - fclk : INTEGER := 50_000_000;

  - -- number of input bits

  - BITS : INTEGER := 8;

  - -- time to debounce in ms

  - t_deb : INTEGER := 20

  - );

  - PORT (

  - clk, rst, pb : IN STD_LOGIC;

  - ssd_out_1, ssd_out_2: OUT STD_LOGIC_VECTOR(6 DOWNTO 0)

  - );

  - END ENTITY toplevel;

  - ARCHITECTURE toplevel_arch OF toplevel IS

  - SIGNAL counter_output_1, counter_output_2 : STD_LOGIC_VECTOR((INTEGER(log2(real(BITS)))) DOWNTO 0);

  - SIGNAL pb_deb : STD_LOGIC;

  - BEGIN

  - debouncer : ENTITY work.switch_debouncer

  - GENERIC MAP(fclk, t_deb)

  - PORT MAP(clk, pb, pb_deb);

  - counter_1 : ENTITY work.counter(on_falling)

  - GENERIC MAP(BITS)

  - PORT MAP(clk => pb_deb, rst => rst, y => counter_output_1);

  - counter_2 : ENTITY work.counter(on_falling)

  - GENERIC MAP(BITS)

  - PORT MAP(clk => pb, rst => rst, y => counter_output_2);

  - ssd_1 : ENTITY work.ssd_interface

  - GENERIC MAP(BITS)

  - PORT MAP(counter_output_1, ssd_out_1);

  - ssd_2 : ENTITY work.ssd_interface

  - GENERIC MAP(BITS)

  - PORT MAP(counter_output_2, ssd_out_2);

  - END ARCHITECTURE toplevel_arch;